###############################################################

Probe Products Corp.

New Demonstration Software for the HSI-24

Version 1.0	07/09/99

###############################################################

Thank you for using the HSI Demonstration Software (hsisrv.exe).

We recommend that you read this file to get the most up-to-date

information about installing, running, and maximizing per-

formance of the HSI Demonstration Software.

For the very latest updates related to the HSI-24, be sure to check out the

Probe Products Corp. Home Page at www.probeproducts.com.

###############################################################

TABLE OF CONTENTS

1) General Information

a) Installing and Uninstalling

b) Support files

2) Startup

a) Run "hsisrv.exe"

b) Setup data

3) File option

Includes:  New,  Open,  Save, Save as, Exit

4) View

Includes:  Numbers, Bar Graphs, Group, Channels, LVDT, Analog, Update

5) Zero

Includes:  Channels, Group

6) Master

Includes:  Channels, Group

7) Setup

Includes:  Channels, Group, LVDT, Analog, General, About

8) DDE (Dynamic Data Exchange) command in Excel

9) How to Contact Probe Products

###############################################################

1) General Information

* This readme.doc file explains the finer points of running the "hsisrv.exe"

application and the support files required.

* We are also working to make the user interface as easy to use as possible.  We

encourage all input from you and will take every suggestion into consideration.

* We have found an unusual occurance that has shown up in this version.  When the

bars are selected, sometimes there are "holes".  This may be easily remedied by

simply changing the size of the window.

a) Installing and Uninstalling

* To install the "HSISRV" program run the program "Setup.exe".

* If you received diskettes this will be found  on Disk 1.

* If you downloaded the "NEWDEMO.ZIP" file from the internet,

UNZIP to a subdirectory of your choice and run the "Setup.exe" file from

there.

* The "Setup.exe" program will copy all the files required automatically.  If any

files already exist and have a newer date than the files being copied, you will

be asked if you want to KEEP your file.  It is suggested that you click YES.  If

you have any questions on this please call Probe Products - software engineering

division.

* To uninstall this program use the install/uninstall option in the control panel.

b) Support files - These files are included in the HSISRV.CAB file and are automatically installed.

* The "HSISRV.EXE" is the application program

* The "PCA10C.BIN" is the firmware for the HSI-24

* DLL files include:

ASYCFILT.DLL, COMCAT.DLL, HSICOM32.DLL,  MFC40.DLL,  MFC42.DLL,

MSVBVM60.DLL,  MSVCRT.DLL,  MSVCRT40.DLL,  OLEAUT32.DLL, OLEPRO32.DLL,

VB6STKIT.DLL

* OCX files used for graphics  include:

COMCTL32.OCX,  COMDLG32.OCX,  GRID32.OCX,  LGAUGE.OCX,  SPIN32.OCX

TABCTL32.OCX

* TLB file used for OLE links STDOLE2.TLB (The OLE links have not been tested yet.)

2) Startup

a) From the START menu in windows the setup program installed HSISRV under the

PROGRAMS item "probeproducts"  unless you chose another at installation.

NOTE: The first time the program is run there will not be an "hsisrv.ini" file.  A message

box will inform you that the default values are being used.

* A menu bar will appear with the options:

File, View, Zero, Master and Setup.

* A button bar appears under the menu for quick access to the following options:

Reset, Zero, Master, Bars and Numbers.

b) Setup data

* The file "hsisrv.ini" contains general setup information.  If no file exists default values

will be set.  The "hsisrv.ini" file will be generated or updated when the program exits.

The setup information may be changed through the SETUP selection on the menu bar.

* The general setup information includes the following:

Maximum number of channels, LVDT's, analog, digital and strain inputs.

It also includes the number of decimals, scan time, ADC averaging, running average,

base address, firmware address and DDE enable status.

* The "hsisrv.ini" file may be viewed with any text editor such as Wordpad.

3) File option

* This option allows for the manipulation of the files with the extension (.hsi).  These

files contain information pertaining to the channel, group, LVDT and analog definitions,

window positioning and general setup information as in the hsisrv.ini file.

* The selections in this option are as follows:

NEW -  Sets the filename to "new.hsi" and clears all data areas.

OPEN - Allows the user to select an preexisting hsi file.

SAVE - Saves the hsi file with the current filename.

SAVE AS - Allows the user to select a new filename or use the current one and then saves

the file.

EXIT - Closes the window and ends the program.  Operates the same way the X at the top

right corner of the window.  However, if the DDE option is in use the program

will notify you and ask if you wish to end the program.

4) View

* This option allows the user to select the type of data to be viewed and which format to view

it in, Number list or Bar Graphs. The Update option updates the display if selected, other wise

it freezes the data displayed.

* The selections in this option are as follows:

Numbers, Bar Graphs, Group, Channels, LVDT, Analog, Update

5) Zero

* This option allows the user to perform the zeroing operation by Channel or by Group

* The selections in this option are as follows:

Channels, Group

6) Master

* This option allows the user to perform the mastering operation by Channel or by Group

* The selections in this option are as follows:

Channels, Group

7) Setup

* The selections in this option are as follows:

Channels, Group, LVDT, Analog, General, About

* An option in the General setup menu allows for DDE links.  Check the box to enable it.

8) DDE (Dynamic Data Exchange) command in Excel

* The DDE command in Excel to establish a link to the data in the HSI-24 program is as follows:

=HSISRV|FormMain!'LabelChan(n)'

where n would be the channel number desired (1..max channel number).

* If the Excel program with DDE links is started and the HSI-24 program (hsisrv.exe)

is not running then Excel will ask if you want to re-establish links.

9) How to Contact Probe Products

If you have any questions about the use or installation of this program,

please contact us:

Probe Products Corporation

1763 Baseline Road

Grand Island, NY     14072-0502

Phone: (716) 773-5554

Software Engineering Division: (914) 229-2764

Fax:   (716) 773-5336

www:   http://www.probeproducts.com

Technical support:

E-mail: ab2ai@compuserve.com Subject: HSI-24 new demo